C81Z-R6 V2.0(6300U)  Touchscreen Computers Datasheet

Configurations

● C81Z-R6 V2.0(6300U): x86-64 / I5-6300U/2.5GHz / 2*SODDR4 / 8G RAM/128G SSD/1TB HDD/WIFI/LTE

Features

● All PCB Board-level interconnection
● Fanless design
● 3x GbE LAN
● 6xRS232/RS485 DB9(COM5-6 support CAN)
● DC9-36 wide range dual input(DC5525+3Px15EDG)
● Legacy VGA port

| Model | C81Z-R6 V2.0(6300U) |  |
| --- | --- | --- |
| Chassis | Mounting | Desktop / Embedded |
| Chipsets | CPU | Integrated I5-6300U/2.5GHz dual-core, 4-thread, TDP 15W |
|  | Graphics | Integrated Intel (R) HD Graphics 620, supports 4K |
|  | Memory | 2*DDR4 SODIMM slots, 2133MHz, up to 32GB |
| Front I/O | Power | 1*Power button, 1*one-key restore, 1*AC loss (power-on boot DIP switch) |
|  | LED | 1*Power LED, 1*HDD LED |
|  | USB | 3*USB2.0, 1*internal USB2.0 |
|  | Terminal | 1*12-pin (4 in / 4 out GPIO, switch, 5V) |
|  | PS2 | 1*PS/2 |
|  | SIM | 1*SIM slot |
| Rear I/O | Display | 1*VGA (1920x1200@60Hz), 1*HDMI1.4 (4096*2304@24Hz) |
|  | LAN | 3*Intel (R) I210AT Gigabit NICs, supports Wake-on-LAN, PXE |
|  | USB | 4*USB3.0 |
|  | COM | 6*RS232/485 DB9, COM5-6 supports CAN output (requires CAN module) |
|  | Audio | 1*Line out, 1*MIC |
|  | Power | 1*DC power jack or 1*3-pin terminal, supports 9~36V input |
| Disk storage | SATA | 1*2.5-inch drive bay |
|  | mSATA | 1*mSATA |
| Expansion | Mini PCIe | 1*Mini PCIe, supports 4G module |
|  | M.2 | 1*M.2/WiFi, Key-E 2230, WiFi module |
| OS | Microsoft Windows | Windows 7/10/11 |
|  | Linux | Ubuntu |
|  | Watchdog | Hardware watchdog reset (256 levels, 0-255 seconds) |
| Power | Power Type | External power adapter or open-frame power supply |
|  | Input Voltage | 9V~36V wide input |
|  | Output Power | 60W~180W |
| Environment | Operating Temperature | -10C to 50C |
|  | Storage Temperature | -20C to 70C |
|  | Relative Humidity | 5%-95% RH, non-condensing |
|  | Operating Vibration | 0.5g rms / 5-500Hz / random / operating |

C81Z-R6 V2.0(6300U)Ordering Information

Front view					Rear view

Dims：MM

Ordering Information

| No. | Model | Technology | Slots Avail. | Configurations | Size | GPIO | WIFI | SIM | LAN | COM | HDMI | VGA | USB | Power |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C81Z-R6 V2.0(5020U) | I3-5020U/2.0GHz | 1*SODDR3L | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 7 | 9~36V |
| 2 | C81Z-R6 V2.0(3685U) | 3865U/1.8GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |
| 3 | C81Z-R6 V2.0(6300U) | I5-6300U/2.5GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |
| 4 | C81Z-R6 V2.0(6650U) | I7-6650U/2.2GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |
| 5 | C81Z-R6 V2.0(7200U) | I5-7200U/2.5GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |
| 6 | C81Z-R6 V2.0 (7500U) | I7-7500U/2.8GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |
| 7 | C81Z-R6 V2.0(8145U) | I3-8145U/2.1GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |
| 8 | C81Z-R6 V2.0(8265U) | I5-8265U/1.6GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |
| 9 | C81Z-R6 V2.0(8565U) | I7-8565U/1.8GHz | 2*SODDR4 | 8G RAM/128G SSD/1TB HDD/WIFI/LTE | x86-64 | 4I4O/PS2/5V/Auto-on |  |  | 3 | 6 | 1 | 1 | 8 | 9~36V |

1. One-key restore / Reset button: select one only. Default: One-key restore.(Some CPUs do not support this feature; FAE confirmation required. Reset button requires hardware change.)
2. I3-5020U CPU: Onboard USB 2.0 or 4G module (select one). Default: 4G module.

Notes: